PORTFOLIO LAB 01

Enterprise Three-Tier Campus Architecture

Cisco Modeling Labs (CML) | Senior Network Engineering Portfolio

Dennis Christenson

Portfolio Project Overview

Purpose: Demonstrate enterprise network architecture, implementation, redundancy, verification, and troubleshooting in a repeatable Cisco Modeling Labs environment. Organized as an engineering build guide so another network engineer can reproduce and validate the design.

Skills Demonstrated

Cisco CML   •   Three-Tier Campus Design   •   Cisco Routing & Switching   •   VLANs / 802.1Q   •   OSPF   •   HSRP   •   Rapid-PVST+   •   Inter-VLAN Routing   •   High Availability   •   Verification & Troubleshooting

CISCO MODELING LABS — TECHNICAL LAB MANUAL SERIES

Cisco Modeling Labs
3-Tier LAN Architecture Lab

Enterprise Campus Network Design:
Core, Distribution & Access Layers

| Version: | 1.0 |
| --- | --- |
| Date: | August 2026 |
| Author: | Dennis |
| Audience: | CCNA / CCNP Students & Network Engineers |
| Skill Level: | Intermediate (CCNA+) |
| Estimated Duration: | 4–6 Hours |
| CML Version: | 2.x or higher |
| Protocols Covered: | OSPF Area 0, HSRP v2, Rapid PVST+, 802.1Q, SSH v2 |

■ AUTHORIZED LAB USE ONLY ■

Table of Contents

1. Lab Overview3

1.1 Learning Objectives3

1.2 Lab Parameters3

2. Architecture Overview4

2.1 Core Layer4

2.2 Distribution Layer4

2.3 Access Layer4

2.4 Redundancy Design4

3. Topology Diagram & Connection Table5

3.1 Logical Topology — ASCII Diagram5

3.2 Physical Connection Table6

4. Device Inventory7

5. IP Addressing Plan8

5.1 Core Point-to-Point Links8

5.2 VLAN SVI Addresses8

5.3 End Host IP Addresses8

5.4 Access Switch Management IPs9

5.5 Loopback Addresses9

6. VLAN Design and Segmentation9

6.1 VLAN Summary9

6.2 Trunk Allowed VLAN Policy10

6.3 HSRP Priority Design10

6.4 Spanning Tree Design10

7. CML Topology Setup Instructions11

7.1 Creating the Topology in CML11

7.2 Verifying Node Boot11

8. Device Configurations12

8.1 CORE-R1 — Cisco ISR432112

8.2 CORE-R2 — Cisco ISR432113

8.3 DIST-SW1 — Cisco Catalyst 365014

8.4 DIST-SW2 — Cisco Catalyst 365016

8.5 ACC-SW1 — Cisco Catalyst 296018

8.6 ACC-SW2 — Cisco Catalyst 296019

8.7 ACC-SW3 — Cisco Catalyst 296020

8.8 ACC-SW4 — Cisco Catalyst 296021

9. End Host (VPC/Desktop) Configuration22

9.1 VPCS — Static IP Assignment22

9.2 Linux Desktop Nodes — IP Assignment22

10. Verification and Troubleshooting23

10.1 Layer 1 & 2 Verification23

10.2 Layer 3 Verification23

10.3 Common Issues and Fixes23

11. Redundancy and Failover Testing24

11.1 HSRP Failover Test24

11.2 OSPF Failover Test24

11.3 STP Failover Test25

12. CML YAML Topology File25

13. Lab Extension Challenges25

14. Quick Reference Card26

14.1 Device Access Summary26

14.2 Key Verification Commands27

14.3 HSRP VIP Quick Reference27

Section 1: Lab Overview

This lab guides you through building a production-quality three-tier enterprise campus LAN inside Cisco Modeling Labs (CML). Every device is fully configured from scratch — no partial configs, no shortcuts. By the end of this lab, you will have a complete, functional enterprise campus network with full redundancy, inter-VLAN routing, dynamic routing, and gateway failover — all verified end-to-end.

1.1 Learning Objectives

Upon completing this lab, you will be able to:

- Design and deploy a hierarchical three-tier LAN: Core, Distribution, and Access layers.

- Configure and verify VLANs across distribution and access layer switches.

- Implement inter-VLAN routing using Layer 3 SVIs on distribution switches.

- Deploy OSPF Area 0 across all routed devices and verify full adjacency formation.

- Configure HSRP version 2 for default gateway redundancy per VLAN.

- Harden Layer 2 with Rapid PVST+, PortFast, BPDU Guard.

- Apply port security best practices on access-layer ports.

- Perform end-to-end connectivity tests including intra-VLAN, inter-VLAN, and core reachability.

- Execute HSRP and OSPF failover tests to validate redundancy design.

1.2 Lab Parameters

| Parameter | Value |
| --- | --- |
| Estimated Duration | 4–6 Hours (including verification) |
| Skill Level | Intermediate — CCNA+ (CCNP recommended) |
| CML Version | 2.x or higher |
| Required Node Types | IOSv (Routers), IOSvL2 / CatalystSW (Switches), Desktop / VPCS (End Hosts) |
| Total Nodes | 16 (2 Core Routers + 2 Dist. Switches + 4 Access Switches + 8 End Hosts) |
| Total Links | 20 (Core P2P + Core-Dist + Dist-Access + Access Hosts) |
| Protocols Used | OSPF Area 0, HSRP v2, Rapid PVST+, 802.1Q, SSH v2 |

⚠ WARNING — CML Resource Requirements This lab requires a CML instance with at least 16 GB RAM and 8 vCPUs available. Each IOSv node requires approximately 512 MB RAM; each IOSvL2 node requires approximately 768 MB. Ensure your CML host meets these requirements before starting. Boot time for all 16 nodes is approximately 4–6 minutes.

Section 2: Architecture Overview

The Cisco three-tier hierarchical model is the gold standard for scalable enterprise campus design. Each layer has a distinct role, ensuring predictable traffic flows, fault isolation, and ease of management. This model separates switching, routing, and aggregation concerns across three clearly defined functional tiers.

2.1 Core Layer

The Core Layer provides high-speed, reliable backbone connectivity between the distribution blocks. In this lab, two Cisco ISR4321 routers (CORE-R1 and CORE-R2) form a fully meshed core with point-to-point /30 links running OSPF Area 0. The core does NOT perform any switching or VLAN functions — it is a pure Layer 3 routing domain.

Key responsibilities: High-speed packet forwarding, OSPF backbone, NTP master/client hierarchy, SSH management plane.

2.2 Distribution Layer

The Distribution Layer is the intelligence layer of the campus network. Two Cisco Catalyst 3650 switches (DIST-SW1 and DIST-SW2) perform inter-VLAN routing via Layer 3 SVIs, implement HSRP v2 for default gateway redundancy, and enforce Spanning Tree root bridge placement via Rapid PVST+.

- DIST-SW1: HSRP Active and STP Root for VLANs 10 and 20

- DIST-SW2: HSRP Active and STP Root for VLANs 30, 40, and 100

2.3 Access Layer

The Access Layer connects end hosts to the network. Four Cisco Catalyst 2960 switches (ACC-SW1 through ACC-SW4) provide 802.1Q-tagged access ports with PortFast, BPDU Guard enabled on all host-facing ports. Each access switch connects via dual 802.1Q trunks to both distribution switches for redundancy.

2.4 Redundancy Design

Every layer is fully redundant — there is no single point of failure in this topology:

- Core: Dual routers, fully meshed /30 P2P links

- Distribution: HSRP v2 active/standby per VLAN, dual uplinks to both core routers

- Access: Dual trunk uplinks to both distribution switches, Rapid PVST+ for loop prevention

Section 3: Topology Diagram & Connection Table

3.1 Logical Topology — ASCII Diagram

+==============================================================+ ¦ CORE LAYER ¦ ¦ +------------------------+ +------------------------+ ¦ ¦ ¦ CORE-R1 ¦ ¦ CORE-R2 ¦ ¦ ¦ ¦ Cisco ISR4321 ¦ ¦ Cisco ISR4321 ¦ ¦ ¦ ¦ Lo0: 10.0.0.1/32 ¦<30>¦ Lo0: 10.0.0.2/32 ¦ ¦ ¦ ¦ G0/0 -> DIST-SW1 ¦ ¦ G0/0 -> DIST-SW1 ¦ ¦ ¦ ¦ G0/1 -> CORE-R2 ¦ ¦ G0/1 -> CORE-R1 ¦ ¦ ¦ ¦ G0/2 -> DIST-SW2 ¦ ¦ G0/2 -> DIST-SW2 ¦ ¦ ¦ +-----------+------------+ +------------+-----------+ ¦ ¦ 172.16.1.1/30 172.16.1.9/30 ¦ ¦ 172.16.1.5/30 172.16.1.13/30 ¦ +=============+==============================+==================+ ¦ L3 P2P (/30) L3 P2P (/30) ¦ v v +==============================================================+ ¦ DISTRIBUTION LAYER ¦ ¦ +------------------------+ +------------------------+ ¦ ¦ ¦ DIST-SW1 ¦<30>¦ DIST-SW2 ¦ ¦ ¦ ¦ Cisco Catalyst 3650 ¦ ¦ Cisco Catalyst 3650 ¦ ¦ ¦ ¦ Lo0: 10.0.0.21/32 ¦ ¦ Lo0: 10.0.0.22/32 ¦ ¦ ¦ ¦ HSRP: V10,V20 ¦ ¦ HSRP: V30,V40,V100 ¦ ¦ ¦ ¦ STP: V10,V20 ¦ ¦ STP: V30,V40,V100 ¦ ¦ ¦ +----+----+----+----+----+ +----+----+----+----+----+ ¦ +======+====+====+====+==============+====+====+====+==========+ ¦ ¦ ¦ 802.1Q Trunks ¦ ¦ ¦ +======+====+====+========================+====+====+===========+ ¦ ACCESS LAYER ¦ ¦ +-----------+ +-----------+ +-----------+ +-----------+ ¦ ¦ ¦ ACC-SW1 ¦ ¦ ACC-SW2 ¦ ¦ ACC-SW3 ¦ ¦ ACC-SW4 ¦ ¦ ¦ ¦ Cat 2960 ¦ ¦ Cat 2960 ¦ ¦ Cat 2960 ¦ ¦ Cat 2960 ¦ ¦ ¦ ¦ VLAN10,20 ¦ ¦ VLAN20,30 ¦ ¦ VLAN30,40 ¦ ¦ VLAN40,100¦ ¦ ¦ +-----+-----+ +-----+-----+ +-----+-----+ +-----+-----+ ¦ +=======+=============+=============+=============+============+ ¦ ¦ ¦ ¦ [Hosts/VPCs] [Hosts/VPCs] [Hosts/VPCs] [Hosts/VPCs] PC-VLAN10-A/B PC-VLAN20-A/B PC-VLAN30-A/B PC-VLAN40-A 10.10.10.100 10.10.20.100 10.10.30.100 10.10.40.100 10.10.10.101 10.10.20.101 10.10.30.101 PC-VLAN100-A 10.10.100.100

3.2 Physical Connection Table

| Link # | From Device | From Port | To Device | To Port | Type | Subnet |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CORE-R1 | G0/1 | CORE-R2 | G0/1 | L3 P2P | 172.16.0.0/30 |
| 2 | CORE-R1 | G0/0 | DIST-SW1 | G0/0 | L3 P2P | 172.16.1.0/30 |
| 3 | CORE-R1 | G0/2 | DIST-SW2 | G0/0 | L3 P2P | 172.16.1.4/30 |
| 4 | CORE-R2 | G0/0 | DIST-SW1 | G0/1 | L3 P2P | 172.16.1.8/30 |
| 5 | CORE-R2 | G0/2 | DIST-SW2 | G0/1 | L3 P2P | 172.16.1.12/30 |
| 6 | DIST-SW1 | G1/0 | ACC-SW1 | G0/1 | 802.1Q Trunk | — |
| 7 | DIST-SW1 | G1/1 | ACC-SW2 | G0/1 | 802.1Q Trunk | — |
| 8 | DIST-SW1 | G1/2 | ACC-SW3 | G0/1 | 802.1Q Trunk | — |
| 9 | DIST-SW1 | G1/3 | ACC-SW4 | G0/1 | 802.1Q Trunk | — |
| 10 | DIST-SW2 | G1/0 | ACC-SW1 | G0/2 | 802.1Q Trunk | — |
| 11 | DIST-SW2 | G1/1 | ACC-SW2 | G0/2 | 802.1Q Trunk | — |
| 12 | DIST-SW2 | G1/2 | ACC-SW3 | G0/2 | 802.1Q Trunk | — |
| 13 | DIST-SW2 | G1/3 | ACC-SW4 | G0/2 | 802.1Q Trunk | — |
| 14 | ACC-SW1 | Fa0/1 | PC-VLAN10-A | eth0 | Access | VLAN 10 |
| 15 | ACC-SW1 | Fa0/2 | PC-VLAN10-B | eth0 | Access | VLAN 10 |
| 16 | ACC-SW1 | Fa0/11 | PC-VLAN20-A | eth0 | Access | VLAN 20 |
| 17 | ACC-SW2 | Fa0/1 | PC-VLAN20-B | eth0 | Access | VLAN 20 |
| 18 | ACC-SW2 | Fa0/11 | PC-VLAN30-A | eth0 | Access | VLAN 30 |
| 19 | ACC-SW3 | Fa0/1 | PC-VLAN30-B | eth0 | Access | VLAN 30 |
| 20 | ACC-SW3 | Fa0/11 | PC-VLAN40-A | eth0 | Access | VLAN 40 |
| 21 | ACC-SW4 | Fa0/11 | PC-VLAN100-A | eth0 | Access | VLAN 100 |

Section 4: Device Inventory

| Device Name | Role | Platform | CML Node Type | RAM | Loopback IP |
| --- | --- | --- | --- | --- | --- |
| CORE-R1 | Core Router | Cisco ISR4321 | IOSv | 512 MB | 10.0.0.1/32 |
| CORE-R2 | Core Router | Cisco ISR4321 | IOSv | 512 MB | 10.0.0.2/32 |
| DIST-SW1 | Distribution Switch | Cisco Catalyst 3650 | IOSvL2 | 768 MB | 10.0.0.21/32 |
| DIST-SW2 | Distribution Switch | Cisco Catalyst 3650 | IOSvL2 | 768 MB | 10.0.0.22/32 |
| ACC-SW1 | Access Switch | Cisco Catalyst 2960 | IOSvL2 | 768 MB | — |
| ACC-SW2 | Access Switch | Cisco Catalyst 2960 | IOSvL2 | 768 MB | — |
| ACC-SW3 | Access Switch | Cisco Catalyst 2960 | IOSvL2 | 768 MB | — |
| ACC-SW4 | Access Switch | Cisco Catalyst 2960 | IOSvL2 | 768 MB | — |
| PC-VLAN10-A | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.10.100/24 |
| PC-VLAN10-B | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.10.101/24 |
| PC-VLAN20-A | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.20.100/24 |
| PC-VLAN20-B | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.20.101/24 |
| PC-VLAN30-A | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.30.100/24 |
| PC-VLAN30-B | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.30.101/24 |
| PC-VLAN40-A | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.40.100/24 |
| PC-VLAN100-A | End Host | Linux Desktop / VPCS | Desktop/VPCS | 512 MB | 10.10.100.100/24 |

Section 5: IP Addressing Plan

5.1 Core Point-to-Point Links (/30)

| Link | Subnet | Device A | IP A | Device B | IP B |
| --- | --- | --- | --- | --- | --- |
| CORE-R1 <-> CORE-R2 | 172.16.0.0/30 | CORE-R1 G0/1 | 172.16.0.1 | CORE-R2 G0/1 | 172.16.0.2 |
| CORE-R1 <-> DIST-SW1 | 172.16.1.0/30 | CORE-R1 G0/0 | 172.16.1.1 | DIST-SW1 G0/0 | 172.16.1.2 |
| CORE-R1 <-> DIST-SW2 | 172.16.1.4/30 | CORE-R1 G0/2 | 172.16.1.5 | DIST-SW2 G0/0 | 172.16.1.6 |
| CORE-R2 <-> DIST-SW1 | 172.16.1.8/30 | CORE-R2 G0/0 | 172.16.1.9 | DIST-SW1 G0/1 | 172.16.1.10 |
| CORE-R2 <-> DIST-SW2 | 172.16.1.12/30 | CORE-R2 G0/2 | 172.16.1.13 | DIST-SW2 G0/1 | 172.16.1.14 |

5.2 VLAN SVI Addresses (Distribution Layer)

| VLAN | Name | Subnet | DIST-SW1 SVI | DIST-SW2 SVI | HSRP VIP | HSRP Active |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Management | 10.10.10.0/24 | 10.10.10.251 | 10.10.10.252 | 10.10.10.254 | DIST-SW1 |
| 20 | Sales | 10.10.20.0/24 | 10.10.20.251 | 10.10.20.252 | 10.10.20.254 | DIST-SW1 |
| 30 | IT-Engineering | 10.10.30.0/24 | 10.10.30.251 | 10.10.30.252 | 10.10.30.254 | DIST-SW2 |
| 40 | Finance | 10.10.40.0/24 | 10.10.40.251 | 10.10.40.252 | 10.10.40.254 | DIST-SW2 |
| 100 | Servers | 10.10.100.0/24 | 10.10.100.251 | 10.10.100.252 | 10.10.100.254 | DIST-SW2 |

5.3 End Host IP Addresses

| Host | VLAN | Switch | Port | IP Address | Gateway |
| --- | --- | --- | --- | --- | --- |
| PC-VLAN10-A | 10 | ACC-SW1 | Fa0/1 | 10.10.10.100/24 | 10.10.10.254 |
| PC-VLAN10-B | 10 | ACC-SW1 | Fa0/2 | 10.10.10.101/24 | 10.10.10.254 |
| PC-VLAN20-A | 20 | ACC-SW1 | Fa0/11 | 10.10.20.100/24 | 10.10.20.254 |
| PC-VLAN20-B | 20 | ACC-SW2 | Fa0/1 | 10.10.20.101/24 | 10.10.20.254 |
| PC-VLAN30-A | 30 | ACC-SW2 | Fa0/11 | 10.10.30.100/24 | 10.10.30.254 |
| PC-VLAN30-B | 30 | ACC-SW3 | Fa0/1 | 10.10.30.101/24 | 10.10.30.254 |
| PC-VLAN40-A | 40 | ACC-SW3 | Fa0/11 | 10.10.40.100/24 | 10.10.40.254 |
| PC-VLAN100-A | 100 | ACC-SW4 | Fa0/11 | 10.10.100.100/24 | 10.10.100.254 |

5.4 Access Switch Management IPs (VLAN 10)

| Device | Management IP | Gateway |
| --- | --- | --- |
| ACC-SW1 | 10.10.10.31/24 | 10.10.10.254 |
| ACC-SW2 | 10.10.10.32/24 | 10.10.10.254 |
| ACC-SW3 | 10.10.10.33/24 | 10.10.10.254 |
| ACC-SW4 | 10.10.10.34/24 | 10.10.10.254 |

5.5 Loopback Addresses

| Device | Loopback0 IP | Purpose |
| --- | --- | --- |
| CORE-R1 | 10.0.0.1/32 | OSPF Router-ID |
| CORE-R2 | 10.0.0.2/32 | OSPF Router-ID |
| DIST-SW1 | 10.0.0.21/32 | OSPF Router-ID |
| DIST-SW2 | 10.0.0.22/32 | OSPF Router-ID |

Section 6: VLAN Design and Segmentation

6.1 VLAN Summary

| VLAN ID | Name | Purpose | Assigned Switches |
| --- | --- | --- | --- |
| 10 | Management | OOB switch management, VLAN 10 SVIs on distribution | All switches |
| 20 | Sales | Sales department end hosts | ACC-SW1 (Fa0/11-20), ACC-SW2 (Fa0/1-10) |
| 30 | IT-Engineering | IT/Engineering end hosts | ACC-SW2 (Fa0/11-20), ACC-SW3 (Fa0/1-10) |
| 40 | Finance | Finance department end hosts | ACC-SW3 (Fa0/11-20), ACC-SW4 (Fa0/1-10) |
| 100 | Servers | Server VLAN | ACC-SW4 (Fa0/11-20) |
| 99 | Native-Trunk | Native VLAN on all trunks (non-default) | All trunk ports |
| 999 | Blackhole-Unused | Unused ports assigned here and shut down | All access switches |

6.2 Trunk Allowed VLAN Policy

| Trunk Link | Allowed VLANs |
| --- | --- |
| DIST-SW1/SW2 <-> ACC-SW1 | 10, 20, 99 |
| DIST-SW1/SW2 <-> ACC-SW2 | 10, 20, 30, 99 |
| DIST-SW1/SW2 <-> ACC-SW3 | 10, 30, 40, 99 |
| DIST-SW1/SW2 <-> ACC-SW4 | 10, 40, 100, 99 |
| DIST-SW1 <-> DIST-SW2 | 10, 20, 30, 40, 100, 99 |

6.3 HSRP Priority Design

| VLAN | DIST-SW1 Priority | DIST-SW2 Priority | Active Router | Preempt Enabled |
| --- | --- | --- | --- | --- |
| 10 | 110 | 90 | DIST-SW1 | Yes (DIST-SW1) |
| 20 | 110 | 90 | DIST-SW1 | Yes (DIST-SW1) |
| 30 | 90 | 110 | DIST-SW2 | Yes (DIST-SW2) |
| 40 | 90 | 110 | DIST-SW2 | Yes (DIST-SW2) |
| 100 | 90 | 110 | DIST-SW2 | Yes (DIST-SW2) |

6.4 Spanning Tree Design

| VLAN | STP Root | STP Secondary | Priority (Root) | Priority (Secondary) |
| --- | --- | --- | --- | --- |
| 10 | DIST-SW1 | DIST-SW2 | 4096 | 8192 |
| 20 | DIST-SW1 | DIST-SW2 | 4096 | 8192 |
| 30 | DIST-SW2 | DIST-SW1 | 4096 | 8192 |
| 40 | DIST-SW2 | DIST-SW1 | 4096 | 8192 |
| 100 | DIST-SW2 | DIST-SW1 | 4096 | 8192 |

Section 7: CML Topology Setup Instructions

7.1 Creating the Topology in CML

Step 1 — Log in to CML

Open a browser and navigate to your CML instance URL (e.g., https://cml-server). Log in with your credentials and click "Add Lab" to create a new lab. Name it: 3-Tier-Enterprise-LAN.

Step 2 — Add Nodes

Add the following nodes by clicking the "+" button and selecting the node type:

- 2x IOSv (name: CORE-R1, CORE-R2)

- 2x IOSvL2 Catalyst 3650 (name: DIST-SW1, DIST-SW2)

- 4x IOSvL2 Catalyst 2960 (name: ACC-SW1, ACC-SW2, ACC-SW3, ACC-SW4)

- 8x Desktop or VPCS (name: PC-VLAN10-A, PC-VLAN10-B, PC-VLAN20-A, PC-VLAN20-B, PC-VLAN30-A, PC-VLAN30-B, PC-VLAN40-A, PC-VLAN100-A)

Step 3 — Connect Nodes

IMPORTANT: In CML, you must RIGHT-CLICK a node and select "Connect" to draw a cable. Simply dragging between nodes does NOT establish a link. Connect all nodes per the connection table in Section 3.2.

Step 4 — Start the Lab

Click "Start Lab" and wait 4–6 minutes for all nodes to boot. Verify all node status indicators turn green before proceeding to apply any configurations.

ℹ NOTE — Virtual Cable Warning CML virtual cables require right-click → Connect to establish the link. Drawing a line visually between nodes does NOT create a working connection. Always verify with show interfaces that connected ports show "up/up" status after booting.

7.2 Verifying Node Boot

Once all nodes are up, open a console to each device and verify the prompt appears. For IOSv routers you will see Router> and for IOSvL2 switches you will see Switch>. Do not begin pasting configurations until all nodes show a clean CLI prompt.

Section 8: Device Configurations

ℹ NOTE — SSH Key Generation After pasting each configuration, run the following command manually on each device to generate the RSA key for SSH: crypto key generate rsa modulus 2048 This cannot be included in the config paste as it requires interactive input. You will be prompted to confirm the key size.

8.1 CORE-R1 — Cisco ISR4321

! ============================================================ ! CORE-R1 — Cisco ISR4321 ¦ Core Layer Router ! OSPF Area 0 ¦ NTP Master ¦ Loopback: 10.0.0.1/32 ! ============================================================ hostname CORE-R1 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ! banner motd ^ ********************************************* * AUTHORIZED ACCESS ONLY -- CORE-R1 * * Unauthorized access is prohibited * ********************************************* ^ ! ! --- Loopback Interface (OSPF Router-ID) --- interface Loopback0 description OSPF-Router-ID-and-Management ip address 10.0.0.1 255.255.255.255 ! ! --- Inter-Core Link to CORE-R2 --- interface GigabitEthernet0/1 description LINK-TO-CORE-R2-G0/1 ip address 172.16.0.1 255.255.255.252 no shutdown ! ! --- L3 Link to DIST-SW1 --- interface GigabitEthernet0/0 description LINK-TO-DIST-SW1-G0/0 ip address 172.16.1.1 255.255.255.252 no shutdown ! ! --- L3 Link to DIST-SW2 --- interface GigabitEthernet0/2 description LINK-TO-DIST-SW2-G0/0 ip address 172.16.1.5 255.255.255.252 no shutdown ! ! --- OSPF Area 0 Configuration --- router ospf 1 router-id 10.0.0.1 passive-interface default no passive-interface GigabitEthernet0/0 no passive-interface GigabitEthernet0/1 no passive-interface GigabitEthernet0/2 network 10.0.0.1 0.0.0.0 area 0 network 172.16.0.0 0.0.0.3 area 0 network 172.16.1.0 0.0.0.3 area 0 network 172.16.1.4 0.0.0.3 area 0 ! ! --- SSH v2 Management --- ip domain-name lab.local ip ssh version 2 ! Run manually: crypto key generate rsa modulus 2048 ! username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous exec-timeout 30 0 line vty 0 4 login local transport input ssh exec-timeout 30 0 ! ! --- NTP Master (Stratum 1 for lab) --- ntp master 1 ! end write memory

8.2 CORE-R2 — Cisco ISR4321

! ============================================================ ! CORE-R2 — Cisco ISR4321 ¦ Core Layer Router ! OSPF Area 0 ¦ NTP Client ¦ Loopback: 10.0.0.2/32 ! ============================================================ hostname CORE-R2 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ! banner motd ^ ********************************************* * AUTHORIZED ACCESS ONLY -- CORE-R2 * * Unauthorized access is prohibited * ********************************************* ^ ! interface Loopback0 description OSPF-Router-ID-and-Management ip address 10.0.0.2 255.255.255.255 ! interface GigabitEthernet0/1 description LINK-TO-CORE-R1-G0/1 ip address 172.16.0.2 255.255.255.252 no shutdown ! interface GigabitEthernet0/0 description LINK-TO-DIST-SW1-G0/1 ip address 172.16.1.9 255.255.255.252 no shutdown ! interface GigabitEthernet0/2 description LINK-TO-DIST-SW2-G0/1 ip address 172.16.1.13 255.255.255.252 no shutdown ! router ospf 1 router-id 10.0.0.2 passive-interface default no passive-interface GigabitEthernet0/0 no passive-interface GigabitEthernet0/1 no passive-interface GigabitEthernet0/2 network 10.0.0.2 0.0.0.0 area 0 network 172.16.0.0 0.0.0.3 area 0 network 172.16.1.8 0.0.0.3 area 0 network 172.16.1.12 0.0.0.3 area 0 ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous exec-timeout 30 0 line vty 0 4 login local transport input ssh exec-timeout 30 0 ! ntp server 10.0.0.1 ! end write memory

8.3 DIST-SW1 — Cisco Catalyst 3650

! ============================================================ ! DIST-SW1 — Cisco Catalyst 3650 ¦ Distribution Layer Switch ! HSRP Active: VLAN 10,20 ¦ STP Root: VLAN 10,20 ! ============================================================ hostname DIST-SW1 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ip routing ! vlan 10 name Management vlan 20 name Sales vlan 30 name IT-Engineering vlan 40 name Finance vlan 100 name Servers vlan 99 name Native-Trunk ! interface Loopback0 description OSPF-Router-ID ip address 10.0.0.21 255.255.255.255 ! interface GigabitEthernet0/0 description LINK-TO-CORE-R1-G0/0 no switchport ip address 172.16.1.2 255.255.255.252 no shutdown ! interface GigabitEthernet0/1 description LINK-TO-CORE-R2-G0/0 no switchport ip address 172.16.1.10 255.255.255.252 no shutdown ! interface GigabitEthernet1/0 description TRUNK-TO-ACC-SW1-G0/1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface GigabitEthernet1/1 description TRUNK-TO-ACC-SW2-G0/1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface GigabitEthernet1/2 description TRUNK-TO-ACC-SW3-G0/1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface GigabitEthernet1/3 description TRUNK-TO-ACC-SW4-G0/1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface Vlan10 description Management-SVI ip address 10.10.10.251 255.255.255.0 standby version 2 standby 10 ip 10.10.10.254 standby 10 priority 110 standby 10 preempt no shutdown ! interface Vlan20 description Sales-SVI ip address 10.10.20.251 255.255.255.0 standby version 2 standby 20 ip 10.10.20.254 standby 20 priority 110 standby 20 preempt no shutdown ! interface Vlan30 description IT-Engineering-SVI ip address 10.10.30.251 255.255.255.0 standby version 2 standby 30 ip 10.10.30.254 standby 30 priority 90 no shutdown ! interface Vlan40 description Finance-SVI ip address 10.10.40.251 255.255.255.0 standby version 2 standby 40 ip 10.10.40.254 standby 40 priority 90 no shutdown ! interface Vlan100 description Servers-SVI ip address 10.10.100.251 255.255.255.0 standby version 2 standby 100 ip 10.10.100.254 standby 100 priority 90 no shutdown ! router ospf 1 router-id 10.0.0.21 passive-interface default no passive-interface GigabitEthernet0/0 no passive-interface GigabitEthernet0/1 network 10.0.0.21 0.0.0.0 area 0 network 10.10.10.0 0.0.0.255 area 0 network 10.10.20.0 0.0.0.255 area 0 network 10.10.30.0 0.0.0.255 area 0 network 10.10.40.0 0.0.0.255 area 0 network 10.10.100.0 0.0.0.255 area 0 network 172.16.1.0 0.0.0.3 area 0 network 172.16.1.8 0.0.0.3 area 0 ! spanning-tree mode rapid-pvst spanning-tree vlan 10,20 priority 4096 spanning-tree vlan 30,40,100 priority 8192 ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous exec-timeout 30 0 line vty 0 15 login local transport input ssh ! ntp server 10.0.0.1 ! end write memory

8.4 DIST-SW2 — Cisco Catalyst 3650

! ============================================================ ! DIST-SW2 — Cisco Catalyst 3650 ¦ Distribution Layer Switch ! HSRP Active: VLAN 30,40,100 ¦ STP Root: VLAN 30,40,100 ! ============================================================ hostname DIST-SW2 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ip routing ! vlan 10 name Management vlan 20 name Sales vlan 30 name IT-Engineering vlan 40 name Finance vlan 100 name Servers vlan 99 name Native-Trunk ! interface Loopback0 description OSPF-Router-ID ip address 10.0.0.22 255.255.255.255 ! interface GigabitEthernet0/0 description LINK-TO-CORE-R1-G0/2 no switchport ip address 172.16.1.6 255.255.255.252 no shutdown ! interface GigabitEthernet0/1 description LINK-TO-CORE-R2-G0/2 no switchport ip address 172.16.1.14 255.255.255.252 no shutdown ! interface GigabitEthernet1/0 description TRUNK-TO-ACC-SW1-G0/2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface GigabitEthernet1/1 description TRUNK-TO-ACC-SW2-G0/2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface GigabitEthernet1/2 description TRUNK-TO-ACC-SW3-G0/2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface GigabitEthernet1/3 description TRUNK-TO-ACC-SW4-G0/2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,40,100,99 no shutdown ! interface Vlan10 description Management-SVI ip address 10.10.10.252 255.255.255.0 standby version 2 standby 10 ip 10.10.10.254 standby 10 priority 90 no shutdown ! interface Vlan20 description Sales-SVI ip address 10.10.20.252 255.255.255.0 standby version 2 standby 20 ip 10.10.20.254 standby 20 priority 90 no shutdown ! interface Vlan30 description IT-Engineering-SVI ip address 10.10.30.252 255.255.255.0 standby version 2 standby 30 ip 10.10.30.254 standby 30 priority 110 standby 30 preempt no shutdown ! interface Vlan40 description Finance-SVI ip address 10.10.40.252 255.255.255.0 standby version 2 standby 40 ip 10.10.40.254 standby 40 priority 110 standby 40 preempt no shutdown ! interface Vlan100 description Servers-SVI ip address 10.10.100.252 255.255.255.0 standby version 2 standby 100 ip 10.10.100.254 standby 100 priority 110 standby 100 preempt no shutdown ! router ospf 1 router-id 10.0.0.22 passive-interface default no passive-interface GigabitEthernet0/0 no passive-interface GigabitEthernet0/1 network 10.0.0.22 0.0.0.0 area 0 network 10.10.10.0 0.0.0.255 area 0 network 10.10.20.0 0.0.0.255 area 0 network 10.10.30.0 0.0.0.255 area 0 network 10.10.40.0 0.0.0.255 area 0 network 10.10.100.0 0.0.0.255 area 0 network 172.16.1.4 0.0.0.3 area 0 network 172.16.1.12 0.0.0.3 area 0 ! spanning-tree mode rapid-pvst spanning-tree vlan 30,40,100 priority 4096 spanning-tree vlan 10,20 priority 8192 ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous exec-timeout 30 0 line vty 0 15 login local transport input ssh ! ntp server 10.0.0.1 ! end write memory

8.5 ACC-SW1 — Cisco Catalyst 2960 (VLAN 10 & 20)

! ============================================================ ! ACC-SW1 — Cisco Catalyst 2960 ¦ Access Layer Switch ! Fa0/1-10 = VLAN 10 (Management) ¦ Fa0/11-20 = VLAN 20 (Sales) ! ============================================================ hostname ACC-SW1 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ! vlan 10 name Management vlan 20 name Sales vlan 99 name Native-Trunk vlan 999 name Blackhole-Unused ! interface Vlan10 description OOB-Management ip address 10.10.10.31 255.255.255.0 no shutdown ! ip default-gateway 10.10.10.254 ! interface range FastEthernet0/1 - 10 description ACCESS-VLAN10-Management switchport mode access switchport access vlan 10 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/11 - 20 description ACCESS-VLAN20-Sales switchport mode access switchport access vlan 20 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/21 - 24 description UNUSED-DISABLED switchport mode access switchport access vlan 999 shutdown ! interface GigabitEthernet0/1 description TRUNK-TO-DIST-SW1-G1/0 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,99 no shutdown ! interface GigabitEthernet0/2 description TRUNK-TO-DIST-SW2-G1/0 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,99 no shutdown ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous line vty 0 15 login local transport input ssh ! ntp server 10.10.10.254 ! end write memory

8.6 ACC-SW2 — Cisco Catalyst 2960 (VLAN 20 & 30)

! ============================================================ ! ACC-SW2 — Cisco Catalyst 2960 ¦ Access Layer Switch ! Fa0/1-10 = VLAN 20 (Sales) ¦ Fa0/11-20 = VLAN 30 (IT-Engineering) ! ============================================================ hostname ACC-SW2 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ! vlan 10 name Management vlan 20 name Sales vlan 30 name IT-Engineering vlan 99 name Native-Trunk vlan 999 name Blackhole-Unused ! interface Vlan10 description OOB-Management ip address 10.10.10.32 255.255.255.0 no shutdown ! ip default-gateway 10.10.10.254 ! interface range FastEthernet0/1 - 10 description ACCESS-VLAN20-Sales switchport mode access switchport access vlan 20 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/11 - 20 description ACCESS-VLAN30-IT-Engineering switchport mode access switchport access vlan 30 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/21 - 24 description UNUSED-DISABLED switchport mode access switchport access vlan 999 shutdown ! interface GigabitEthernet0/1 description TRUNK-TO-DIST-SW1-G1/1 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,99 no shutdown ! interface GigabitEthernet0/2 description TRUNK-TO-DIST-SW2-G1/1 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,20,30,99 no shutdown ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous line vty 0 15 login local transport input ssh ! ntp server 10.10.10.254 ! end write memory

8.7 ACC-SW3 — Cisco Catalyst 2960 (VLAN 30 & 40)

! ============================================================ ! ACC-SW3 — Cisco Catalyst 2960 ¦ Access Layer Switch ! Fa0/1-10 = VLAN 30 (IT-Engineering) ¦ Fa0/11-20 = VLAN 40 (Finance) ! ============================================================ hostname ACC-SW3 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ! vlan 10 name Management vlan 30 name IT-Engineering vlan 40 name Finance vlan 99 name Native-Trunk vlan 999 name Blackhole-Unused ! interface Vlan10 description OOB-Management ip address 10.10.10.33 255.255.255.0 no shutdown ! ip default-gateway 10.10.10.254 ! interface range FastEthernet0/1 - 10 description ACCESS-VLAN30-IT-Engineering switchport mode access switchport access vlan 30 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/11 - 20 description ACCESS-VLAN40-Finance switchport mode access switchport access vlan 40 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/21 - 24 description UNUSED-DISABLED switchport mode access switchport access vlan 999 shutdown ! interface GigabitEthernet0/1 description TRUNK-TO-DIST-SW1-G1/2 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,30,40,99 no shutdown ! interface GigabitEthernet0/2 description TRUNK-TO-DIST-SW2-G1/2 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,30,40,99 no shutdown ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous line vty 0 15 login local transport input ssh ! ntp server 10.10.10.254 ! end write memory

8.8 ACC-SW4 — Cisco Catalyst 2960 (VLAN 40 & 100)

! ============================================================ ! ACC-SW4 — Cisco Catalyst 2960 ¦ Access Layer Switch ! Fa0/1-10 = VLAN 40 (Finance) ¦ Fa0/11-20 = VLAN 100 (Servers) ! ============================================================ hostname ACC-SW4 ! no ip domain-lookup service password-encryption enable secret Cisco@123 ! vlan 10 name Management vlan 40 name Finance vlan 100 name Servers vlan 99 name Native-Trunk vlan 999 name Blackhole-Unused ! interface Vlan10 description OOB-Management ip address 10.10.10.34 255.255.255.0 no shutdown ! ip default-gateway 10.10.10.254 ! interface range FastEthernet0/1 - 10 description ACCESS-VLAN40-Finance switchport mode access switchport access vlan 40 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/11 - 20 description ACCESS-VLAN100-Servers switchport mode access switchport access vlan 100 spanning-tree portfast spanning-tree bpduguard enable no shutdown ! interface range FastEthernet0/21 - 24 description UNUSED-DISABLED switchport mode access switchport access vlan 999 shutdown ! interface GigabitEthernet0/1 description TRUNK-TO-DIST-SW1-G1/3 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,40,100,99 no shutdown ! interface GigabitEthernet0/2 description TRUNK-TO-DIST-SW2-G1/3 switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan 10,40,100,99 no shutdown ! ip domain-name lab.local ip ssh version 2 username admin privilege 15 secret Cisco@123 ! line con 0 logging synchronous line vty 0 15 login local transport input ssh ! ntp server 10.10.10.254 ! end write memory

Section 9: End Host (VPC/Desktop) Configuration

9.1 VPCS — Static IP Assignment

For nodes using the VPCS node type in CML, open the console for each VPC and run the following commands:

NOTE: you will need to save these commands under the config tab when you click on the device.  They will not be saved if you do not do this.

! PC-VLAN10-A (ACC-SW1, Fa0/1, VLAN 10) PC-VLAN10-A> ip 10.10.10.100 255.255.255.0 10.10.10.254 PC-VLAN10-A> save ! PC-VLAN10-B (ACC-SW1, Fa0/2, VLAN 10) PC-VLAN10-B> ip 10.10.10.101 255.255.255.0 10.10.10.254 PC-VLAN10-B> save ! PC-VLAN20-A (ACC-SW1, Fa0/11, VLAN 20) PC-VLAN20-A> ip 10.10.20.100 255.255.255.0 10.10.20.254 PC-VLAN20-A> save ! PC-VLAN20-B (ACC-SW2, Fa0/1, VLAN 20) PC-VLAN20-B> ip 10.10.20.101 255.255.255.0 10.10.20.254 PC-VLAN20-B> save ! PC-VLAN30-A (ACC-SW2, Fa0/11, VLAN 30) PC-VLAN30-A> ip 10.10.30.100 255.255.255.0 10.10.30.254 PC-VLAN30-A> save ! PC-VLAN30-B (ACC-SW3, Fa0/1, VLAN 30) PC-VLAN30-B> ip 10.10.30.101 255.255.255.0 10.10.30.254 PC-VLAN30-B> save ! PC-VLAN40-A (ACC-SW3, Fa0/11, VLAN 40) PC-VLAN40-A> ip 10.10.40.100 255.255.255.0 10.10.40.254 PC-VLAN40-A> save ! PC-VLAN100-A (ACC-SW4, Fa0/11, VLAN 100) PC-VLAN100-A> ip 10.10.100.100 255.255.255.0 10.10.100.254 PC-VLAN100-A> save

9.2 Linux Desktop Nodes — IP Assignment

For nodes using the Linux Desktop node type in CML, open the terminal on each desktop and run the following commands (as root or with sudo):

## PC-VLAN10-A — VLAN 10 ip addr flush dev eth0 ip addr add 10.10.10.100/24 dev eth0 ip link set eth0 up ip route add default via 10.10.10.254 ## PC-VLAN10-B — VLAN 10 ip addr flush dev eth0 ip addr add 10.10.10.101/24 dev eth0 ip link set eth0 up ip route add default via 10.10.10.254 ## PC-VLAN20-A — VLAN 20 ip addr flush dev eth0 ip addr add 10.10.20.100/24 dev eth0 ip link set eth0 up ip route add default via 10.10.20.254 ## PC-VLAN20-B — VLAN 20 ip addr flush dev eth0 ip addr add 10.10.20.101/24 dev eth0 ip link set eth0 up ip route add default via 10.10.20.254 ## PC-VLAN30-A — VLAN 30 ip addr flush dev eth0 ip addr add 10.10.30.100/24 dev eth0 ip link set eth0 up ip route add default via 10.10.30.254 ## PC-VLAN30-B — VLAN 30 ip addr flush dev eth0 ip addr add 10.10.30.101/24 dev eth0 ip link set eth0 up ip route add default via 10.10.30.254 ## PC-VLAN40-A — VLAN 40 ip addr flush dev eth0 ip addr add 10.10.40.100/24 dev eth0 ip link set eth0 up ip route add default via 10.10.40.254 ## PC-VLAN100-A — VLAN 100 ip addr flush dev eth0 ip addr add 10.10.100.100/24 dev eth0 ip link set eth0 up ip route add default via 10.10.100.254

Section 10: Verification and Troubleshooting

10.1 Layer 1 & 2 Verification

| Verification Goal | Command | Expected Output |
| --- | --- | --- |
| Interface is connected | show interfaces status | All connected ports show "connected" |
| VLAN database | show vlan brief | VLANs 10, 20, 30, 40, 99, 100, 999 active |
| Trunk ports | show interfaces trunk | Trunk ports listed with allowed VLANs |
| STP root bridge | show spanning-tree vlan 10 | DIST-SW1 shown as root for VLAN 10 |
| STP port roles | show spanning-tree | Designated / Root / Alternate per port |

10.2 Layer 3 Verification

| Verification Goal | Command | Expected Output |
| --- | --- | --- |
| OSPF neighbors | show ip ospf neighbor | All neighbors in FULL state |
| OSPF routes | show ip route ospf | O and O IA routes for all subnets |
| HSRP status | show standby brief | Active / Standby state per VLAN |
| Inter-VLAN ping | ping 10.10.20.100 source vlan10 | Success from distribution switch |
| End-to-end ping | ping 10.10.30.100 from PC-VLAN10-A | 5/5 packets succeed |

10.3 Common Issues and Fixes

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Interface shows "notconnect" | CML cable not properly connected | Right-click node → Connect; verify with show int status |
| Trunk shows no VLANs | switchport trunk encapsulation missing | Add: switchport trunk encapsulation dot1q |
| OSPF neighbor stuck in INIT | Passive-interface on wrong port | Check: no passive-interface on uplink ports |
| HSRP in SPEAK state | Preempt not configured | Add: standby X preempt on higher-priority switch |
| Ping fails between VLANs | ip routing not enabled on distribution | Add: ip routing under global config |
| PC can't reach gateway | Default gateway not set | Set ip default-gateway on access switch; verify PC gateway |
| Cross-VLAN ping fails but local works | Return path issue | Check remote host default gateway and HSRP active router |

Section 11: Redundancy and Failover Testing

11.1 HSRP Failover Test

Step 1 — Verify Initial HSRP State

Run on both DIST-SW1 and DIST-SW2. Confirm DIST-SW1 is Active for VLAN 10 and DIST-SW2 is Active for VLAN 30.

DIST-SW1# show standby brief DIST-SW2

# show standby brief

Step 2 — Start Continuous Ping from PC-VLAN10-A

# Windows: ping 10.10.30.100 -t

# Linux: ping 10.10.30.100

Step 3 — Shut Down DIST-SW1 VLAN 10 SVI

DIST-SW1# configure terminal

DIST-SW1(config)#

interface Vlan10

DIST-SW1(config-if)#

shutdown

Step 4 — Observe HSRP Failover

DIST-SW2 should become Active for VLAN 10 within 10 seconds (hold timer expiry). Ping drops should be minimal — typically 1–3 packets during the transition window.

Step 5 — Restore and Verify Preemption

Issue no shutdown on the VLAN 10 SVI. DIST-SW1 should preempt back to Active within the hold timer period due to the configured preempt delay.

DIST-SW1(config-if)#

no shutdown

DIST-SW1(config-if)

# end

DIST-SW1# show standby brief

11.2 OSPF Failover Test

Step 1 — Verify OSPF Neighbors on CORE-R1

CORE-R1# show ip ospf neighbor

Step 2 — Shut Down the Link to DIST-SW1

CORE-R1# configure terminal

CORE-R1(config)# interface GigabitEthernet0/0

CORE-R1(config-if)# shutdown

Step 3 — Verify OSPF Reconvergence

OSPF should reconverge through CORE-R2. All routes to distribution subnets should remain reachable via the alternate path. Default convergence occurs within 40 seconds (dead interval). Rapid OSPF timers can be configured to reduce this.

CORE-R1# show ip route ospf

CORE-R1# show ip ospf neighbor

Step 4 — Restore the Link

CORE-R1(config-if)# no shutdown

CORE-R1(config-if)# end

CORE-R1# show ip ospf neighbor

11.3 STP Failover Test

Step 1 — Verify STP Root on ACC-SW1

Confirm the root port points toward DIST-SW1 (STP root for VLAN 10).

ACC-SW1# show spanning-tree vlan 10

Step 2 — Shut Down the Trunk to DIST-SW1

ACC-SW1# configure terminal

ACC-SW1(config)# interface GigabitEthernet0/1

ACC-SW1(config-if)# shutdown

Step 3 — Verify STP Reconvergence

The port toward DIST-SW2 should transition from Alternate to Root port state. With Rapid PVST+, convergence should complete within 1–2 seconds.

ACC-SW1# show spanning-tree vlan 10

ℹ NOTE — Restore After Failover Tests Always restore all interfaces to their original state with no shutdown after completing each failover test before proceeding to the next test scenario. Verify with show standby brief, show ip ospf neighbor, and show spanning-tree that the network has fully converged back to its nominal state.

Section 12: CML YAML Topology File

ℹ NOTE — YAML Import Instructions The YAML topology file can be used to import this lab directly into CML without manually placing and connecting nodes. To use it: 1. Navigate to CML Lab Manager → Import → Paste YAML 2. Paste the complete YAML content and click Import 3. The file includes all node placements, interface mappings, and link definitions for all 16 nodes and 20 links 4. After successful import, start the lab and wait for all nodes to boot 5. Note: Interface addresses and all device configurations still need to be applied manually after import — the YAML file defines topology only, not IOS configurations

The YAML topology file is available as a companion file to this lab manual. It is named 3-Tier-Enterprise-LAN.yaml and should be stored alongside this document. If you are building the lab manually, use the connection table in Section 3.2 as your reference and the YAML file is not required.

Section 13: Lab Extension Challenges

After completing the base lab, the following extension challenges will deepen your understanding of enterprise network design. Each challenge is independent and can be completed in any order.

Add a Firewall (ASAv or FTDv)
 Insert a Cisco ASAv between the core layer and a simulated "Internet" node. Configure NAT overload, zone-based policies, and basic inspection rules. Verify that inside hosts can reach outside while outside cannot initiate sessions inward. Document your access-list and NAT translations using show conn and show xlate.

Implement DHCP on Distribution Switches
 Configure DHCP server pools on DIST-SW1 and DIST-SW2 for each VLAN. Remove static IP assignments from end hosts and verify they receive addresses dynamically. Configure DHCP relay (ip helper-address) on SVIs pointing to the authoritative DHCP server. Verify with show ip dhcp binding.

Add BGP at the Core Layer
 Connect CORE-R1 and CORE-R2 to simulated ISP routers. Configure eBGP peering, advertise the 10.10.0.0/16 summary route, and implement basic BGP policy with local preference and route filtering using prefix lists. Verify with show bgp summary and show ip route bgp.

Implement QoS on Access Ports
 Configure DSCP marking and queuing policies on access switch uplink ports. Mark VoIP traffic (VLAN 10) with EF (DSCP 46), business applications with AF41 (DSCP 34), and default traffic with Best Effort. Apply service policies on uplink interfaces and verify with show policy-map interface and show mls qos interface.

Deploy Network Management (NMS)
 Add a simulated NMS server in VLAN 100 (Servers). Configure SNMPv3 with authentication and privacy on all devices, set up syslog with severity level 6 (informational) to the NMS, and verify trap reception. Optionally add a NetFlow collector and configure flow export on the distribution switches using ip flow-export.

Convert to EIGRP
 Replace OSPF with EIGRP on all Layer 3 devices using named EIGRP mode. Configure MD5 authentication between all EIGRP peers. Compare convergence times and routing table output between OSPF and EIGRP. Document the differences in neighbor formation timers, metric calculation, and feasibility conditions.

Section 14: Quick Reference Card

■ CISCO 3-TIER LAB — QUICK REFERENCE

14.1 Device Access Summary

| Device | Console | Management IP | Username | Password | Enable |
| --- | --- | --- | --- | --- | --- |
| CORE-R1 | Direct | 10.0.0.1 (Lo0) | admin | Cisco@123 | Cisco@123 |
| CORE-R2 | Direct | 10.0.0.2 (Lo0) | admin | Cisco@123 | Cisco@123 |
| DIST-SW1 | Direct | 10.10.10.251 (VLAN 10) | admin | Cisco@123 | Cisco@123 |
| DIST-SW2 | Direct | 10.10.10.252 (VLAN 10) | admin | Cisco@123 | Cisco@123 |
| ACC-SW1 | Direct | 10.10.10.31 | admin | Cisco@123 | Cisco@123 |
| ACC-SW2 | Direct | 10.10.10.32 | admin | Cisco@123 | Cisco@123 |
| ACC-SW3 | Direct | 10.10.10.33 | admin | Cisco@123 | Cisco@123 |
| ACC-SW4 | Direct | 10.10.10.34 | admin | Cisco@123 | Cisco@123 |

14.2 Key Verification Commands

| Purpose | Command |
| --- | --- |
| Verify all interfaces | show interfaces status |
| Verify VLAN database | show vlan brief |
| Verify trunks | show interfaces trunk |
| Verify OSPF neighbors | show ip ospf neighbor |
| Verify routing table | show ip route |
| Verify HSRP | show standby brief |
| Verify spanning tree | show spanning-tree vlan 10 |
| Save configuration | write memory |
| Verify SSH | show ip ssh |
| Check NTP sync | show ntp status |

14.3 HSRP VIP Quick Reference

| VLAN | Name | HSRP VIP | Active | Standby |
| --- | --- | --- | --- | --- |
| 10 | Management | 10.10.10.254 | DIST-SW1 | DIST-SW2 |
| 20 | Sales | 10.10.20.254 | DIST-SW1 | DIST-SW2 |
| 30 | IT-Engineering | 10.10.30.254 | DIST-SW2 | DIST-SW1 |
| 40 | Finance | 10.10.40.254 | DIST-SW2 | DIST-SW1 |
| 100 | Servers | 10.10.100.254 | DIST-SW2 | DIST-SW1 |

CISCO MODELING LABS — 3-TIER LAN ARCHITECTURE LAB | Version 1.0 | August 2026 | Author: Dennis
 AUTHORIZED LAB USE ONLY — For CCNA / CCNP Educational Purposes | Riverview, FL
 This document was prepared for use with Cisco Modeling Labs (CML) 2.x and later.